# How to upgrade the Router Firmware

The Router has a flash chip that can be upgraded with a new firmware.  Please read the instructions below to upgrade the firmware.

Note: You must have firmware version 1116 or above to upgrade.

- Double click on Tftp to install the Upgrade Firmware software.

- Server- Enter the IP Address of the router that you assigned.  By default, the router’s IP Address is 192.168.1.1 as shown above.

- Password- Enter the password you assigned the router. By default, the router’s password is “admin”.

- File- Click the “…” button to browse for the firmware file with the BIN extension that was part of the extracted file you downloaded.  In the example, the xxx.bin file was extracted on the C Drive.

- Click Upgrade button to start upgrading.  A progress bar will appear to show the progress of the upgrade. When successful, click Close and close the window. When the Diag LED stops flashing, the upgrade is complete.

Note: During the upgrade, please do not power down the Router.

Upgrade is complete